# LBP11 Lime Putty			         		     		Health & Safety Data Sheet

| ## Product Names | LBP11.01 Super Fine Lime Putty LBP11.02 Chalk Lime Putty |
| --- | --- |
| ## Description | Calcium Hydroxide in water. |
| Dry Analysis (Typical) | Calcium Hydroxide [Ca(OH)2]>94.0% |
| REACH Data | CAS No 1305-62-0. EINECS No 215-137-3. Alkali (pH 13). Danger: H318 Causes serious eye damage. Warning: H315 Causes skin irritation. STOT SE3 H335 May cause respiratory irritation. |
| ## UUID (REACH IT) | ECHA-d2b2a357-1ad0-4665-ae39-5ce27671975e |
| ## Pre-Reg REACH IT | 17-2119409270-53-0000 |
| ## Storage | Store in original containers in cool environment. Protect from heat and frost. Unlimited shelf life. Lime Putty matures with time and will stiffen in the tub. |
| ## Precautionary Statements | P102 Keep out of reach of children. P280 Wear protective gloves, eye protection/face protection. P305 + P351 + P310 If in eyes rinse cautiously with water for several minutes and immediately get medical assistance. P302 + P352 If on skin, wash affected parts immediately with soap and water. P501 Dispose of contents/containers at hazardous waste collection point. |
| Protective Measures | Suitable protective clothing is essential. Eye Protection – goggles/face protection designed to protect against liquid splashes. |
| Emergency First Aid | Oral Ingestion: Wash mouth with water and drink plenty of water. Do not induce vomiting. Seek medical attention immediately. Eye Contact: SPEED IS ESSENTIAL. Irrigate immediately with plenty of clean water for at least 15 minutes. May cause serious damage to eyes if left untreated. Seek medical attention immediately. Skin Contact: Immediately wash with water for at least 15 minutes. Remove contaminated clothing and launder before re-use. Seek medical attention if irritation persists. May cause dermatitis or alkaline burns. |
| Spills | Scoop up. Lime putty and lime mortar on floors is a slipping hazard. |
| Disposal | Dispose of as non-toxic waste at a hazardous waste collection point. Do not allow to enter drains or watercourses. |
| Fire Fighting | Non-flammable. No known harmful decomposition products. |
| Transport | Not dangerous for transport. Transport in upright sealed containers. |
|  | Technical Department on 01460 281921 Email: info@limebase.co.uk Limebase Products Ltd Walronds Park Web: www.limebase.co.uk Isle Brewers Taunton Somerset TA3 6QP All product data and H&S sheets can be printed from our web site |